Title

Evaluation of a High-Throughput Method for Processing Sponge-Stick Samples to Detect Viable, Non-Spore-Forming Biothreat Agents

Authors

Vanessa L. Brissona, Staci R. Kanea, Michael Worth Calfeeb, Stephen Cendrowskic, Sanjiv R. Shahd

Affiliations

a Biosciences and Biotechnology Division, Physical and Life Sciences Directorate, Lawrence Livermore National Laboratory, Livermore, CA, USA

b Homeland Security and Materials Management Division, Center for Environmental Solutions and Emergency Response, Office of Research and Development, U.S. Environmental Protection Agency, Research Triangle Park, NC, USA

c Chemical, Biological, Radiological, Nuclear Consequence Management Advisory Division

Office of Emergency Management, U.S. Environmental Protection Agency, Washington, DC, USA

d Homeland Security and Materials Management Division, Center for Environmental Solutions and Emergency Response, Office of Research and Development, U.S. Environmental Protection Agency, Washington, DC, USA

Corresponding Author

Sanjiv R. Shah, 1200 Pennsylvania Ave, NW, 8801R, US Environmental Protection Agency, Office of Research and Development, Washington, DC, 20460, USA.

E-mail address: shah.sanjiv@epa.gov

Declarations of interest

None.

Abstract

After a bioterrorism incident, surface sampling is often used to determine the extent of contamination and exposure, guiding decontamination efforts and decisions for re-occupancy of affected sites. The sponge-stick (SS) is a preferred and commonly used device for sample collection to detect both spore-forming and non-spore-forming biothreat agents from non-porous surfaces. A recently developed high-throughput method (HTM) for processing SS samples to detect viable Bacillus anthracis spores was adapted for detection of non-spore-forming biothreat agents, Yersinia pestis and Francisella tularensis. The scalable HTM was used to process up to 20 SS samples simultaneously, compared to the current stomacher-based method which processes one SS at a time. Comparisons of the HTM and the stomacher-based method were statistically indistinguishable for most experiments (P > 0.05) with HTM recoveries of 37-60% for Y. pestis inoculated at 102-103 cells/SS and held 48 hours at 4°C to mimic sample transport/storage. The HTM was integrated with Rapid Viability-Polymerase Chain Reaction (RV-PCR) analysis to detect viable Y. pestis in the presence of particulate contamination (Arizona Test Dust, ATD). This approach detected Y. pestis inoculated at 20 cells/SS and ATD did not impact detection (P > 0.05). F. tularensis showed significantly lower recoveries between no-hold time and 48-hour hold time (4°C, P < 0.05) using the HTM, which further testing showed could be due to toxicity of the neutralizing buffer used for SS pre-wetting. With modifications, this method could enhance throughput capacity while maintaining similar recovery efficiencies to current methods for other non-spore-forming bacterial pathogens.

Keywords

Yersinia pestis, Francisella tularensis, high-throughput sample processing, sponge-stick samples, vegetative bacterial cells, cell viability

Highlights

- A high-throughput method (HTM) for sponge-stick (SS) sample processing was adapted for a non-spore-forming biothreat agent.

- The HTM can simultaneously process 20 SS samples in 2-3 hours, compared to 4-5 hours for the stomacher-based method.

- HTM recovery of Y. pestis was comparable to the traditional stomacher-based method (~40% recovery) for low cell levels (102 cells/SS).

- The HTM was integrated with Rapid Viability-Polymerase Chain Reaction (RV-PCR) analysis, detecting as few as ~20 Y. pestis cells/SS.

- F. tularensis recovery was poor (≤ 32%) due in part to incompatibility with the SS pre-wetting buffer.

1. Introduction

The sponge-stick is a preferred and commonly used device for collection of samples from non-porous surfaces to detect the presence of spores of Bacillus anthracis, the causative agent for anthrax (CDC, 2012) and assess the extent of contamination and decontamination efficacy.  The national validation of the sponge-stick processing method established sponge-stick-based sampling as a reliable method for B. anthracis spores (Rose et al., 2011).  In this study, sponge-sticks were processed one at a time in a sterile, plastic Stomacher bag with large volume of buffer (90 mL) using a Stomacher 400 Circulator (Seward, Bohemia, NY).  The method was later updated to use 45 mL buffer for spore recovery, although the other steps remained unchanged (Chan-Riley et al., 2024).  We recently developed a HTM for simultaneously processing multiple sponge-stick samples to detect viable B. anthracis spores (Brisson et al., 2025) that can help expedite analysis of a large number of samples expected from a potential wide-area anthrax bioterrorism incident.  The HTM using only 35 mL of extraction buffer gave B. anthracis Sterne spore recoveries (57 – 74%) which is comparable to that from the traditional stomacher-based sponge-stick processing method (56 – 79%) (Brisson et al., 2025).  Given the high level of B. anthracis persistence under various environmental stressors (heat, desiccation, UV irradiation, etc.) due to its spore formation (Setlow, 2006; 2007), this study was conducted to evaluate the HTM for recovery of less resilient, non-spore-forming biothreat agents such as Y. pestis and F. tularensis.

Both Y. pestis and F. tularensis are Tier 1 Select Agents that pose a severe threat to human health (CDC, 2025) and are a bioterrorism risk to national security.  Y. pestis, the bacterium that causes plague, was responsible for multiple pandemics (Riedel, 2005) including more recent outbreaks between 2017 and 2024 in Africa (World Health Organization, 2017; CBRNe World, 2024).  The bacterium is considered a high-consequence biological weapon (Inglesby et al., 2000) which could be especially dangerous if used in an aerosol attack (Hoffman and Norton, 2000).  F. tularensis causes tularemia, a serious debilitating disease, with a low infective dose (1–10 cells; Jones et al., 2005; Saslaw et al., 1961) and high morbidity and mortality.  These features led this bacterium to be considered a high-consequence biological weapon (Dennis et al., 2001) with national security concerns over its potential use as a bioterrorism agent (Oyston et al., 2004; Mierzynska et al., 2002; Nelson and Sjöstedt, 2024).

Both Y. pestis and F. tularensis have been shown to survive for several days on various surfaces under certain environmental conditions (Wilkinson, 1966; Rose et al., 2003; US EPA, 2010), including persistence to 99 days under low temperature and low humidity conditions (US EPA, 2015).  However, since environmental persistence is significantly lower for these agents compared to Bacillus spores that can persist for several years, fewer studies have focused on surface sampling methods for non-spore forming pathogens.  Aslett et al. (2024) recently evaluated different sponge-stick types for recovery of surrogates (Escherichia coli, Yersinia ruckeri, and Serratia marcescens) of non-spore forming pathogens (Brucella abortus, B. melitensis, B. suis, Vibrio cholerae, and Y. pestis) from non-porous surfaces using a Stomacher for sample processing and observed higher cell recovery efficiencies for the Solar-Cult® sponge-stick compared to the San-StickÒ or 3MÔ sponge-wipes, with the least variability across surface types (stainless steel, plastic, and glass).  In addition, the Solar-Cult® sponge-stick has a quick sponge release handle which is preferred by field sampling personnel.  Therefore, using this sampling device, we adapted the high-throughput 50-mL tube vortexing method for sponge-stick samples (Brisson et al., 2025) for non-spore forming biothreat agents, using avirulent and live vaccine strains, and compared this method with the traditional stomacher-based method (Rose et al., 2011; US EPA, 2016) for cell recovery.  This study also showed that the high throughput method could be readily applied to Y. pestis cells—with comparable results between both sample processing methods—while additional method modification/optimization will be required for its application to F. tularensis cells.

2. Materials and Methods

2.1 Y. pestis and F. tularensis Cultures and Growth Media

Y. pestis strain K25 Derivative 72 (D72; Cat. No. NR-4697) and F. tularensis subsp. holarctica Live Vaccine Strain (LVS; Cat. No. NR-646) were obtained from Biodefense & Emerging Infections Research Resources Repository (Manassas, VA).  For liquid cultivation, Y. pestis K25 D72, an avirulent strain lacking the pCD1 plasmid, was grown on Y. pestis Enrichment Broth (YPEB) (EPA, 2016; Kane et al., 2019), and the avirulent F. tularensis (LVS) was grown in Brain Heart Infusion Broth with Vitox/Fildes/Histidine supplements (BVFH) prepared as described previously (Morris et al., 2017; Shah, 2019).  For cultivation on solid medium, Y. pestis was grown on Tryptose Blood Agar Base (TBAB) plates without blood prepared from Difco TBAB powder (Becton Dickinson, Cat. No. 223220) and F. tularensis was grown on Chocolate Agar (CA) plates (Hardy Diagnostics, Cat. No. E14BX).

Freezer stocks were prepared for each strain by first reviving the strain in liquid media and dilution plating on solid media to obtain isolated colonies.  For each strain, a single colony was collected on an inoculation loop, inoculated into liquid media, and incubated overnight (18 to 24 hours) at 28°C and 37°C for Y. pestis and F. tularensis, respectively.  The overnight cultures were aliquoted into 2-mL cryotubes (0.5 mL of culture in each).  Next, 0.5 mL of 30% (by volume) glycerol in water was added and the tube was mixed gently prior to storing at -80 °C.  These freezer stocks were used for all subsequent experiments.

2.2 High-throughput Cell Recovery

2.2.1. Impacts of Vortexing on Cell Viability

Since the high throughput sponge-stick processing method was originally developed for spores (Brisson et al., 2025) and vegetative cells are more susceptible to damage than spores, we first conducted experiments to determine the impact of vortexing on cell viability by inoculating cells into buffer (Phosphate Buffered Saline with 0.05% Triton X-100 [PBSTX] for Y. pestis, and phosphate buffered saline [PBS] for F. tularensis) and vortexing at either half maximum speed or maximum speed for 5 minutes (3 replicates per speed) and comparing viable cell counts to non-vortexed controls (3 replicates).

2.2.2 Preparation of Sponge-Sticks

To prepare inoculum for each experiment, one aliquot of the appropriate freezer stock was thawed.  Two 50-mL tubes were prepared with 4.5 mL liquid growth media, and each tube was inoculated with 0.5 mL of the thawed freezer stock.  Inoculated tubes were incubated at 28°C for Y. pestis and 37°C F. tularensis with shaking (200 RPM) overnight (18 to 24 hours). The optical density at 600 nm (OD600) of the overnight culture was measured, and the culture was diluted in buffer (PBSTX for Y. pestis, and either PBSTX or PBS for F. tularensis) to achieve an OD600 of approximately 0.10.  PBSTX was tested for swab samples containing Y. pestis cells showing good maintenance of cell viability (US EPA, 2012; Gilbert et al., 2014); however, this buffer had not been tested with F. tularensis previously.  This cell suspension was further diluted to achieve the desired inoculum concentration (101 to 104 cells per 100 µL depending on the experiment).  To determine the inoculum concentration for each experiment, the inoculum cell suspensions, or dilutions thereof, were plated in triplicate on agar plates (100 µL per plate, TBAB for Y. pestis and CA for F. tularensis) and incubated at the appropriate temperature.  Colonies were counted after either 48 hours (Y. pestis) or 72 hours (F. tularensis) to determine the inoculum levels as colony forming units (CFU) per SS.

Sponge-sticks (Solar-Cult®, Hardy Diagnostics, Cat. No. SH10NB1; prewet with 10-mL Neutralizing Buffer) were inoculated by adding 100 µL of cell suspension across both sides of the sponge in a dropwise manner (50 µL on each sponge side).  For some experiments Arizona Test Dust (ATD) was added to samples to simulate interference from environmental sampling.  A 10 mg/mL suspension of non-sterile ATD was prepared in sterile water, and 200-µL of this suspension (containing 2 mg ATD) was distributed across each sponge in a dropwise manner (with ~100-µL per side).  Prepared sponges were deposited in sterile specimen cups, and 5 mL of Neutralizing Buffer (NB) was added to each cup to minimize cell desiccation. Specimen cups were closed, sealed with parafilm, and stored in a sealed plastic bag at 4°C for approximately 48 hours to simulate the expected holding time between sponge-stick sample collection and processing for analysis. For some experiments, additional sponges were prepared and processed immediately (without the 48-hour hold time) in order to assess the impacts of hold time on recovery of viable cells. For all experiments, 3 to 5 replicate sponges were prepared for each experimental condition, and at least 2 negative control (NC) sponges were also prepared.

2.2.3. Sponge-Stick Processing

Sponge-sticks were processed using a modified high throughput processing method originally developed for recovery of B. anthracis spores (Brisson et al, 2025) with the following modifications. First, different extraction buffers were used because the buffer used for recovering B. anthracis spores (70% Phosphate buffered saline with 0.05% Tween 20 [PBST], 30% ethanol) was not suitable for recovering viable vegetative cells.  Instead, phosphate-buffered saline with 0.05% Triton X-100 (PBSTX) was used for recovery of Y. pestis and phosphate buffered saline (PBS) was used for recovery of F. tularensis; PBSTX was shown to inhibit F. tularensis growth (data not shown).  Additionally, the initial cell extraction buffer volume was reduced from 25 mL to 20 mL to accommodate the 5 mL NB added to the sponges in the specimen cups to help maintain higher moisture conditions.  After the sponge was transferred to the 50 mL tube for the first extraction, the remaining NB in the specimen cup was transferred to the same tube.

2.2.4. Viable Cell Recovery Analysis

Recovery of viable cells was quantified by serial dilution plating of the recovered cell suspension using the same buffer as that for cell recovery (PBSTX or PBS depending on the experiment).  Dilutions were plated in triplicate (100 µL per plate) onto the appropriate solid medium (TBAB for Y. pestis; CA for F. tularensis), and plates were incubated (28 °C for Y. pestis; 37 °C F. tularensis).  After incubation (48 hours for Y. pestis; 72 hours for F. tularensis), colonies were counted to determine CFU and calculate recovery percentages.

In addition, Rapid Viability – Polymerase Chain Reaction (RV-PCR) analysis was integrated as an alternative analysis method providing higher throughput and more rapid results than culture analysis. RV-PCR was conducted only with Y. pestis because of poor recovery of F. tularensis in initial experiments (see Results).  While RV-PCR is a qualitative method, a 101-cell level (10–99 cells) sensitivity of detection was demonstrated for virulent Y. pestis in water samples even with complex backgrounds (PCR inhibitors, metal oxides, background microbes) and in the presence of dead target cells (Kane et al., 2019).  Here, we used RV-PCR to analyze the cell suspension recovered from sponge-stick sample processing.  Specifically, the volume of the recovered cell suspension was measured, and PBSTX was added to increase the sample volume to 2.7 mL. In a sterile 48 well deep-well plate, one well per sample was prepared with 0.3 mL of sterile 10xYPEB. Samples were transferred to the prepared wells and mixed gently with the pipette tip. A T0 sample (500 µL) from each well was immediately collected and centrifuged at 14,000 rpm at 4°C for 10 min to pellet the cells. Supernatant (300 µL) was removed, and the remaining sample was frozen and stored at -20 °C until DNA extraction (up to 1 week). The well plate was sealed with a Breath-Easy membrane and incubated at 28 °C with shaking (200 rpm) for 24 hours.  After incubation, the plate was unsealed and two T24 samples (500 µL each) were collected from each well and centrifuged at 14,000 rpm at 4°C for 10 min to pellet the cells. Supernatant (300 µL) was removed, and the remaining sample was frozen and stored -20 °C until DNA extraction.  DNA was extracted using the Promega MagneSil® kit (Promega, Cat. No. MD1360) with modifications to the manufacturer’s protocol as previously described (Shah, 2017; Brisson et al., 2025).

The chromosomal assay YC2 was used on triplicates of each undiluted sample or NC DNA extract (Kane et al., 2019), with T0 and T24 aliquots from the same sample or NC analyzed on the same PCR plate.  Each plate included 10-fold serial dilutions (5 ng to 5 pg genomic DNA) of Y. pestis Strain K25 D72 genomic DNA obtained from BEI Resources (Cat. No. NR-4721, Lot No. 57988653) and a no-template control (Nuclease-Free Water).  Each DNA dilution (5 mL) was added to 20 mL of PCR mix containing the following: 12.5 ml of TaqMan 2´ Universal PCR Master Mix (Applied Biosystems, Cat. No. 4305719), 0.4 mL of 4 mM probe, 0.5 mL of 10 mM each for the forward and reverse primers, and 6.1 mL Nuclease-Free Water (Invitrogen).  PCR was performed on T0 and T24 DNA extracts using the ABI 7500 Fast platform (Applied Biosystems) in fast mode with the following parameters: 2 min at 50°C, 10 min at 95°C, followed by 45 amplification cycles (5 s at 95°C for denaturation and 20 s at 60°C for annealing/extension) as previously established (Kane et al., 2019).  Three replicate PCR analyses were conducted per sample or NC extract replicate. The assay primers and probe for the YC2 chromosomal assay were (5’ to 3’):

- YC2 Forward Primer: CAACGACTAGCCAGGCGAC

- YC2 Reverse primer: CATTGTTCGCACGAAACGTAA

- YC2 Probe: FAM- TTTTATAACGATGCCTACAACGGCTCTGCAA-BHQ1

2.3. Conventional Stomacher Method for Sponge-Stick Processing

The Stomacher method for sponge-stick samples (EPA, 2016) was followed using a Stomacher® 400 Circulator (Seward Cat. No. 0400/001/AJ).  Briefly, each inoculated sponge was transferred with forceps to a Stomacher bag (Seward, Cat. No. BA6141/CLR) and any remaining liquid in the specimen cup was also transferred to the corresponding bag.  Then, 90 mL of sterile PBST was added to the bag using 50-mL serological pipets, the top of the stomacher bag was closed using the whirl-pack type closure, and the bag was placed into the Stomacher.  The bag was processed at 200 rpm for 1 min at room temperature and then removed and left to settle for 10 min to allow the foam to dissipate.  The sponge was then removed from the bag with a sterile forceps after squeezing out as much liquid as possible.  The bag contents were then mixed by 50-mL serological pipet and transferred to two 50-mL conical tubes.

To concentrate the cells, tubes were centrifuged at 4,000 RPM (3,220 ´ g) for 30 minutes at 4 °C.  The supernatant was carefully removed and discarded leaving the cell pellet and approximately 3 mL of buffer in each tube. The tubes were vortex-mixed on high setting for 2 min in 10-sec bursts to suspend the cell pellets.  The suspensions from the two tubes were combined into one tube and the volume was measured and recorded. The cell suspensions were plated on TBAB plates to determine CFUs as described for the HTM above.

2.4 Data analysis

Cell counts (CFU) or percent recovery are reported as the average ± the standard deviation (SD) for three to five biological replicates per sample replicate.  Data for all individual replicates are included in the data tables. Within each experiment, two-tailed Student’s t-tests (two-sample equal variance) were used to test for significant differences between treatments in Microsoft Excel.  Individual P-values are shown, or the P-value is expressed as > 0.05 (statistically insignificant) or < 0.05 (statistically significant). When analyzing data from multiple replicate experiments, an analysis of variance (ANOVA) was conducted in R, using experiment number as an additional categorical variable.

For the statistical analysis of replicate culture or PCR analyses per sample replicate, the overall standard deviation from all sample replicates was calculated using the following equation,

Overall∨jointSD=ini−1si2+ni×Xi−X2ini−1

where ni = the number of cell counts or PCR analyses per sample for sample replicate i; si = the SD of the cell count or PCR (CT) values for the individual sample i; Xi = the average cell count or CT value for the individual sample i; X = the overall average cell count or CT value for the samples.

3. Results

3.1.  HTM for Processing Y. pestis Inoculated Sponge Sticks

3.1.1. Impacts of Vortexing on Cell Viability

We found that the viable cell recovery was comparable (P > 0.05 for all pairwise comparisons) between no vortexing (5500 ± 1380 CFU), vortexing at half of maximum speed (5020 ± 2100 CFU), and vortexing at the maximum speed (5440 ± 790 CFU) (Table 1).  This experiment was repeated at a higher cell concentration (14200 ± 4000, 15800 ± 3000, and 14700 ± 2200 CFU for no vortexing, half speed, and full speed vortexing, respectively) with similar results (P > 0.05 for all pairwise comparisons).  Based on these results, we selected the maximum vortexing speed for subsequent experiment with Y. pestis.

3.1.2. Recovery from Sponge-Sticks at Different Hold Times

We conducted experiments to determine the recovery of Y. pestis at the 103-cell level. The cell recovery was 80% ± 14% (1660 ± 290 CFU) for the samples processed immediately after inoculation and 37% ± 7% (760 ± 150 CFU) for the samples processed after 48 hours (Table 2).  This drop in viable cell recovery from the 0-hour to 48-hour samples was statistically significant (P = 0.011).

This experiment was replicated with similar results, although the replicate experiment did not show statistically significant differences between the 0-hour and 48-hour samples (P = 0.12).  The inoculum concentrations for the replicate experiment was 1780 ± 160 per SS and the recovery was 61% ± 12% (1090 ± 210 CFU) for the samples processed immediately after inoculation and 80% ± 15% (1430 ± 260 CFU) for the samples processed after 48 hours (Table 2). Because the results of the first two experiments were variable (different trends and statistical significance in one experiment but not in another), we conducted a third experiment with inoculum level of 1,350 ± 80 CFU/SS and higher replication (5 replicates per hold time).  In this experiment, the recovery was 90% ± 26% (1210 ± 350 CFU) for samples processed immediately and 67% ± 19% (900 ± 260 CFU) for samples processed after 48 hours (Table 3).  These differences were not statistically significant (P = 0.052).

Because the results were variable between replicated experiments, we conducted an ANOVA of the combined data, which indicated that there was a small but statistically significant (P = 0.007) reduction in recovery with the 48-hour hold-time (62% ± 20% vs. 80% ± 24%).  Based on these results, and the expectation that real world samples would require a transport time of approximately 48 hours, all subsequent experiments for recovery of Y. pestis cells from SS were conducted with a 48-hour hold time.

3.1.2. Comparison of High-throughput and Traditional Stomacher Sample Processing Methods

We compared recovery of viable Y. pestis cells from sponge-sticks between the HTM and the traditional stomacher-based processing method at two different inoculum concentrations, 102- and 103-cell levels. All samples were processed after a 48-hour hold time at 4°C. The 102-cell level was 340 ± 30 CFU/SS and the 103-cell level was 3410 ± 320 CFU/SS.  For the 102-cell level, the recoveries of the two methods were not statistically different (P = 0.99). The high throughput method recovered 40% ± 5% (136 ± 16 CFU) and the stomacher method recovered 40% ± 1% (135 ± 4 CFU) (Table 3).  For the 103-cell level, the high throughput method recovered 37% ± 9% (1250 ± 290 CFU) and the stomacher method recovered 52% ± 9% (1780 ± 320 CFU) (Table 3).  This difference was statistically significant (P = 0.045).

This experiment was replicated with similar results.  In the replicate experiment the 102-cell level was 190 ± 15 CFU/SS, and the 103-cell level was 1900 ± 150 CFU/SS.  For the 102-cell level, the recoveries of the two methods were not statistically different (P = 0.31). The HTM recovered 39% ± 7% (74 ± 14 CFU) and the stomacher method recovered 54% ± 26% (103 ± 50 CFU) (Table 3).  For the 103-cell level, the high throughput method recovered 48% ± 8% (920 ± 150 CFU) and the stomacher method recovered 72% ± 10% (1370 ± 260 CFU) (Table 6). This difference was statistically significant (P = 0.0056).

To further evaluate the two methods given this statistically significant result (Table 3), the comparison experiment was replicated a third time. In this third experiment, the 102-cell level was 371 ± 12 CFU/SS and the 103-cell level was 3710 ± 120 CFU/SS.  For the 102-cell level , the recoveries of the two methods were not statistically different (P = 0.13). The HTM recovered 40% ± 4% (150 ± 14 CFU) and the stomacher method recovered 52% ± 12% (192 ± 46 CFU) (Table 3).  For the 103-cell level, the HTM recovered 60% ± 6% (2230 ± 230 CFU) and the stomacher method recovered 67% ± 12% (2500 ± 460 CFU) (Table 3). This difference was not statistically significant (P = 0.30).

Because there was some variability between results of replicate experiments, we conducted an ANOVA of the combined data. This analysis indicated that there was no significant difference between the HTM and the stomacher method for the 102-cell level (P = 0.07), but for the 103-cell level there was statistically (P = 0.0015) lower recovery with the HTM compared to the stomacher method (49% ± 12% vs. 64% ± 17%).

3.1.3 Integration with RV-PCR for Detection from Complex Samples

We conducted an experiment to integrate our high throughput sponge-stick processing method with the RV-PCR analysis method in the presence of non-sterile Arizona test dust (ATD) as an environmental interference matrix (Table 4).  This experiment was conducted at two inoculum levels: 220 ± 20 CFU/SS and 19 ± 6 CFU/SS. For the 102-cell level, the average DCT values (difference between CT of T0 and T24 for given SS sample) were 11.7 ± 1.9 and 13.2 ± 2.1 for samples with and without ATD, respectively.  For the 101-cell level, the average DCT values were 14.5 ± 1.5 and 11.4 ± 4.2 for samples with and without ATD, respectively.  A positive detection result is indicated by DCT ³ 6. All individual samples except one (a 101-cell level sample without ATD) met this threshold for positive detection, and all negative controls yielded non-detect results.  There was no statistical difference between DCT values with and without ATD for either inoculum level (P = 0.439 for the 102-cell level and P = 0.150 for the 101-cell level).

This experiment was replicated with similar results (Table 5).  The inoculum levels in the replicate experiment were 235 ± 22 CFU/SS and 22 ± 7 CFU/SS. For the 102-cell level, the average DCT values (difference between CT values for T0 and T24 aliquots for a given SS sample) were 16.4 ± 2.7 and 15.6 ± 1.9 for samples with and without ATD, respectively.  For the 101-cell level, the average DCT values were 11.0 ± 4.3 and 11.3 ± 3.3 for samples with and without ATD, respectively.  As with the previous experiment, there was no statistical difference between DCT values with and without ATD for either inoculum level (P = 0.759 for the 102-cell level and P = 0.926 for the 101-cell level). These data show that RV-PCR analysis can be integrated with our high throughput SS processing method for Y. pestis and that ATD did not negatively affect RV-PCR analysis.

3.2. HTM for Processing F. tularensis Inoculated Sponge Sticks

3.2.1. Impacts of Vortexing on Cell Viability

We conducted experiments to determine the impacts of vortexing on cell viability by diluting F. tularensis cells in PBS buffer and vortexing at either maximum speed or half maximum speed for 5 minutes and comparing viable cell counts to non-vortexed controls. The final viable cells recovered were comparable (P > 0.05 for all comparisons) between no vortexing (15400 ± 1600 CFU), vortexing at half maximum speed (16400 ± 1400 CFU), and vortexing at maximum speed (15800 ± 1500 CFU) (Table 6).  This experiment was replicated with similar results (11700 ± 1100, 11800 ± 1800, and 11900 ± 1200 CFU for no vortexing, half speed, and full speed vortexing) (P >. 0.05 for all pairwise comparisons).  Based on these results, we selected the maximum vortexing speed for subsequent experiments with F. tularensis.

3.2.2. Recovery from Sponge-Sticks at Different Hold Times

We conducted experiments to determine the recovery of F. tularensis cells added to sponges at different inoculum levels and processed either immediately after SS inoculation (0 hour hold time) or after 48 hours at 4 °C.  The inoculum level for this experiment was 2130 ± 80 CFU/SS and cell recovery was 83% ± 10% (1760 ± 220 CFU) for the samples processed immediately after inoculation and 32% ± 20% (670 ± 420 CFU) for the samples processed after 48 hours (Table 7).  The drop in recovery from the 0-hour to 48-hour samples was statistically significant (P = 0.016).

This experiment was replicated with similar results.  The inoculum level for the replicate experiment was 1880 ± 250 CFU/SS and recovery was 67% ± 11% (1270 ± 200 CFU) for the samples processed immediately after inoculation and 9% ± 7% (160 ± 140 CFU) for the samples processed after 48 hours (Table 7).  This drop in recovery from the 0-hour to 48-hour hold-time samples was again statistically significant (P = 0.0005).

3.2.1. Impacts of Different Buffers on Cell Viability

Because the viability of the F. tularensis cells dropped dramatically with the 48-hour hold time at 4 °C, we conducted another experiment to determine whether the NB (as pre-wetting buffer with the SS and added here to maintain moisture during the hold time) was negatively impacting cell viability. For this experiment, we inoculated F. tularensis cells into 10 mL of either NB or PBS and determined viable cell concentrations immediately and after a 48-hour hold time at 4 °C.  The inoculum level for this experiment was 2080 ± 250 CFU.  For samples in NB, viable cell recoveries were 2210 ± 690 CFU and 1400 ± 470 CFU for the 0-hour and 48-hour hold times, respectively (Table 8).  For samples in PBS, viable cell recoveries were 2540 ± 690 CFU and 2180 ± 530 CFU for the 0-hour and 48-hour hold times, respectively.  The reduced recovery after 48-hours at 4 °C was statistically significant in NB (P < 0.0001) but was not statistically significant in PBS (P = 0.45).  This experiment was replicated with similar results.  The inoculum level for the replicate experiment was 2270 ± 290 CFU. For samples in NB, viable cell recoveries 2070 ± 360 CFU and 1170 ± 360 CFU for the 0-hour and 48-hour hold times, respectively.  For samples in PBS, viable cell recoveries were 2520 ± 490 CFU and 2340 ± 630 CFU for the 0-hour and 48-hour hold times, respectively.  The reduced recovery after 48-hours was statistically significant in NB (P = 0.011) but was not statistically significant in PBS (P = 0.24).

4. Discussion

Here, we describe a high-throughput sample processing method for detection of Y. pestis cells and other non-spore-forming bacterial pathogens from the sponge-stick sampling device that was adapted from that for B. anthracis spores (Brisson et al., 2025).  The method is scalable and could process 24 sponge-stick samples simultaneously (2-3 hours total processing time per sample set) using a multi-plate vortexer compared to one sponge-stick at a time for the current stomacher-based method (4-5 hours total processing time per sample set). Use of an orbital shaker (along with a higher capacity centrifuge) as described for B. anthracis spores (Brisson et al., 2025), could further enhance sample throughput. In comparison testing (ANOVA), the HTM showed similar cell recoveries to the stomacher-based method using Y. pestis cells added to sponge-sticks with no statistically significant differences between cell recovery percentages for the 102-cell level and slightly reduced recoveries for the 103-cell level.

The same features of the B. anthracis high-throughput sample processing method apply here including significantly lower extraction buffer volume used (35-mL compared to 90-mL for the stomacher-based method) resulting in less generated waste and shorter centrifugation times (i.e., fewer tubes for centrifugation), and thus shorter overall processing time. Furthermore, there is no need for specialized equipment (i.e., Stomacher), and standard laboratory equipment could be used for sample processing, either a multi-plate vortexer or orbital shaker.

Experiments were also conducted to determine the effect of holding Y. pestis-spiked sponge-sticks for 48-hours at 4°C to mimic sample shipment and storage prior to analysis. Sponge-sticks come pre-wet with 10-mL NB (from the vendor) for surface sampling.  Here, after adding Y. pestis cells to sponges and placing them in specimen cups, 5-mL NB was added to the cup to maintain moist conditions and cell viability.  This buffer addition prior to sample storage is not currently included in the protocol for Y. pestis sampling (US EPA, 2016), although specimen cups used for sample collection may be pre-filled with NB.  Both NB and PBSTX (used for extraction) were shown to be compatible with Y. pestis cell viability maintenance for swab samples (Gilbert et al., 2014). However, unlike swab samples, sponges in specimen cups could not be submerged in buffer to maintain saturation conditions, likely causing more variability in Y. pestis cell recovery in this study.  In general, lower cell recovery was observed after the 48-hour hold; however, results were not statistically significant, and one experiment showed the opposite trend.

While the sponge-stick sample processing method could be readily adapted for Y. pestis (avirulent D72 strain) cells, our preliminary results showed that additional method modifications would be required to obtain similar performance for F. tularensis cells.  F. tularensis (LVS) cell recovery from sponges after storage at 4 °C for 48-hours was poor with values ranging from 5% to 32%.  An initial trouble-shooting experiment showed that NB as a pre-wetting and storage buffer for sponges likely contributed to some cell viability loss due to toxicity, although additional changes may be needed to keep sponges saturated in a compatible buffer prior to processing.  Placing sponges directly into PBS transport medium may help maintain F. tularensis cell viability; however, this may be challenging during field sampling, sample transport, and storage prior to analysis.

5. Conclusions

In summary, this effort represents significant improvements in throughput capacity, ease of use, and time for processing for our HTM compared with the traditional stomacher method with similar Y. pestis cell recovery efficiency.  Furthermore, the high throughput method was used effectively as the front-end processing protocol for RV-PCR analysis with no observed effect of ATD on sensitivity of detection. Future testing could assess method performance with higher levels and different types of environmental interferents.  Preliminary results with F. tularensis LVS cells showed that method modifications are required to maintain cell viability toward development of such a high-throughput sample processing method.  These results suggested that each non-spore-forming bacterial pathogen may have specific requirements for method optimization.  Higher throughput methods such as that described here will help expedite analysis of a large number of samples expected from a potential wide-area bioterrorism incident to rapidly assess contamination extent and pathogen exposure risk.

Funding information

The United States Environmental Protection Agency through its Office of Research and Development funded the research described here under an Interagency Agreement (EPA IA DW- DW-089-92552401-1).  Lawrence Livermore National Laboratory is operated by Lawrence Livermore National Security (LLNS), LLC, for the U.S. Department of Energy, National Nuclear Security Administration under Contract DE-AC52-07NA27344.

CRediT authorship contributions

Vanessa L. Brisson’s contributions included Methodology, Investigation, Validation, Data curation, Formal analysis, Visualization, Writing – original draft, and Writing – review and editing.

Staci R. Kane contributions included Methodology, Investigation, Data curation, Formal analysis, Writing – original draft, and Writing – review and editing.

Michael Worth Calfee’s contributions included Data curation, and Writing – review and editing.

Stephen Cendrowski’s contributions included Data curation, and Writing – review and editing.

Sanjiv R. Shah’s contributions included Conceptualization, Funding acquisition, Project administration, Data curation, Writing – original draft, and Writing – review and editing.

Acknowledgments

We thank Brian McMinn (EPA) and Asja Korajkic (EPA) for technical review and Ramona Sherman (EPA) for QA review of this manuscript.

Disclaimer

The views expressed in this article are those of the authors and do not necessarily represent the views or policies of the U.S. Environmental Protection Agency.  Mention of trade names or commercial products does not constitute endorsement or recommendation for use.  The DOE contractor role did not include establishing Agency policy.

References

- Aslett LD, Calfee MW, Monge M, Abdel-Hady A, Chamberlain T, Baartmans R, Touati A. 2024. Evaluation of sponge wipe surface sampling for collection of potential surrogates for non-spore-forming bioterrorism agents. J Appl Microbiol. 135(5): lxae09. https://doi.org/10.1093/jambio/lxae097

- Brisson VB, Kane SR, Shah SR. 2025. Development of a high-throughput method for processing sponge-stick samples to detect viable Bacillus anthracis spores.  J Microbiol Methods. (in review).

- CBRNe World. 2024. An unusual number of suspected cases of plague with significant fatality in DRC.  Accessed 5/6/25. https://cbrneworld.com/news/an-unusual-number-of-suspected-cases-of-plague-with-significant-fatality-in-drc.

- Centers for Disease Control and Prevention (CDC). 2012. Emergency Response Resources: Surface sampling procedures for Bacillus anthracis spores from smooth, non-porous surfaces. Centers for Disease Control and Prevention: Atlanta, GA. https://www.cdc.gov/niosh/anthrax/php/surface-sampling/?CDC_AAref_Val=https://www.cdc.gov/niosh/topics/emres/surface-sampling-bacillus-anthracis.html

- Centers for Disease Control and Prevention and US Department of Agriculture (CDC/UDSA). 2025. HHS and USDA Select Agents and Toxins. Federal Select Agent Program. https://www.selectagents.gov/sat/docs/HHS-USDA-Select-Agents-and-Toxins-List_508.pdf

- Chan-Riley MY, Edwards JR, Noble-Wang J, Rose L. 2024. Investigating surface area and recovery efficiency of healthcare-associated pathogens to optimize composite environmental sampling. PLoS ONE 19(11): e0310283. https://doi.org/10.1371/journal.pone.0310283

- Dennis DT, Inglesby TV, Henderson DA, Bartlett JG, Ascher MS, Eitzen E, Fine AD, Friedlander AM, Hauer J, Layton M, Lillibridge SR, McDade JE, Osterholm MT, O'Toole T, Parker G, Perl TM, Russell PK, Tonat K; Working Group on Civilian Biodefense. 2001.Tularemia as a biological weapon: medical and public health management. JAMA. 285(21):2763-73. doi: 10.1001/jama.285.21.2763.

- Gilbert SE, Rose LJ, Howard M, Bradley MD, Shah S, Silvestri E, Schaefer FW 3rd, Noble-Wang. 2014.  Evaluation of swabs and transport media for the recovery of Yersinia pestis.  J Microbiol Methods. 96:35-41. doi: 10.1016/j.mimet.2013.10.017.

- Hoffman RE, Norton JE. 2000. Lessons learned from a full-scale bioterrorism exercise. Emerg Infect Dis. 6:652–3. doi: 10.3201/eid0606.000617.

- Inglesby TV, Dennis DT, Henderson DA, Bartlett JG, Ascher MS, Eitzen E, Fine AD, Friedlander AM et al. 2000. Plague as a biological weapon: medical and public health management. Working Group on Civilian Biodefense. JAMA 283, 2281–2290.

- Jones RM, Nicas M, Hubbard A, et al., 2005. The Infectious Dose of Francisella tularensis (Tularemia). Applied Biosafety, 10(4) pp. 227-239.

- Kane SR, Shah SR, Alfaro TM. 2019. Development of a rapid viability polymerase chain reaction method for detection of Yersinia pestis. J Microbiol Meth. 162:21-27. https://doi.org/10.1016/j.mimet.2019.05.005

- Mierzyńska D, Hermanowska-Szpakowicz T. 2002. Tularemia jako potencjalna broń bioterrorystów [Tularemia as a potential weapon of bioterrorists]. Med Pr. 53(3):279-81.

- Morris BJ, Buse HY, Adcock NJ, Rice EW. 2017. A novel broth medium for enhanced growth of Francisella tularensis. Lett. Appl. Microbiol. 64, 394–400. https://doi.org/10.1111/lam.12725.

- Nelson CA, Sjöstedt A. Tularemia: A Storied History, An Ongoing Threat. Clin Infect Dis. 2024 Jan 31;78(Suppl 1):S1-S3. doi: 10.1093/cid/ciad681.

- Oyston PC, Sjöstedt A, Titball RW. 2004. Tularaemia: bioterrorism defence renews interest in Francisella tularensis. Nature Reviews Microbiology. 2(12):967-78.

- Riedel S. Plague: from natural disease to bioterrorism. Proc (Bayl Univ Med Cent). 2005 Apr;18(2):116-24. doi: 10.1080/08998280.2005.11928049.

- Rose LJ, Donlan R, Banerjee SN, Arduino MJ. 2003. Survival of Yersinia pestis on environmental surfaces. Appl Environ Microbiol., 69(4):2166-71. https://doi.org/10.1128/AEM.69.4.2166-2171.200

- Rose LJ, Hodges L, O’Connell H, Noble-Wang J. 2011. National Validation Study of a Cellulose Sponge Wipe-Processing Method for Use after Sampling Bacillus anthracis Spores from Surfaces. Appl Environ Microbiol 77: https://doi.org/10.1128/AEM.05377-11

- Rose LJ, Houston H, Martinez-Smith M, Lyons AK, Whitworth C, Reddy SC, et al. 2022. Factors influencing environmental sampling recovery of healthcare pathogens from non-porous surfaces with cellulose sponges. PLoS ONE 17(1): e0261588. https://doi.org/10.1371/journal. Pone.0261588.

- Saslaw S, Eigelsbach, HT, Prior, JA, Wilson, HE, et al., 1961. Tularemia vaccine study II. Respiratory challenge. Archives of Internal Medicine,107, 702-714.

- Shah S. 2019. Protocol for Detection Francisella tularensis from Environmental Samples During the Remediation Phase of a Tularemia Incident”. EPA/600/R-19/110 | September 2019. https://cfpub.epa.gov/si/si_public_record_Report.cfm?dirEntryId=348592&Lab=NHSRC

- Setlow P. 2006. Spores of Bacillus subtilis: their resistance to radiation, heat and chemicals. J. Appl. Microbiol. 101, 514–525.

- Setlow P. 2007) I will survive: DNA protection in bacterial spores. TRENDS in Microbiology 15(4): 172-180. doi:10.1016/j.tim.2007.02.004.

- Sinclair R, Boone SA, Greenberg D, Keim P, Gerba CP. 2008. Persistence of Category A Select Agents in the Environment. Appl Environ Microbiol 74, 555-563. https://doi.org/10.1128/AEM.02167-07.

- U.S. EPA. 2010. Persistence Testing and Evaluation of Fumigation Technologies for Decontamination of Building Materials Contaminated with Biological Agents. U.S. Environmental Protection Agency, Washington, DC, EPA/600/R-10/086.

- U.S. EPA. 2015. Determination of the Persistence of Non-Spore-Forming Biological Threat Agents in the Environment, U.S. Environmental Protection Agency, Washington, DC, EPA/600/R-15/303.

- U.S. EPA. 2016. Protocol for Detection Yersinia pestis from Environmental Samples During the Remediation Phase of a Plague Incident”. EPA/600/R-16/109 August 2016. https://cfpub.epa.gov/si/si_public_record_report.cfm?Lab=NHSRC&dirEntryId=329170

- Wilkinson TR. 1966. Survival of bacteria on metal surfaces. Appl Microbiol. 14(3):303-7. https://doi.org/10.1128/am.14.3.303-307.1966

- World Health Organization, 2017. https://www.who.int/emergencies/disease-outbreak-news/item/15-november-2017-plague-madagascar-en

Tables

Table 1. Recovery of viable Y. pestis cells after vortexing at different speeds.

| Experiment | Condition | Total CFUs Recovered Avg. ± SDa | Pairwise T-test P-values Compared to: |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | No Speed | Half Speed | Top Speed |
| Initial Experiment | No (Speed) Vortexing | 5610 ± 1510 | NA | 0.58 | 0.92 |
|  |  | 5770 ± 440 |  |  |  |
|  |  | 5100 ± 2190 |  |  |  |
|  | Avg. ± Overall SD | 5500 ± 1380 |  |  |  |
|  | Half Speed Vortexing | 7530 ± 1400 | 0.58 | NA | 0.76 |
|  |  | 3350 ± 950 |  |  |  |
|  |  | 4180 ± 150 |  |  |  |
|  | Avg. ± Overall SD | 5020 ± 2100 |  |  |  |
|  | Top Speed Vortexing | 5190 ± 810 | 0.92 | 0.76 | NA |
|  |  | 5690 ± 730 |  |  |  |
|  |  | 5440 ± 1050 |  |  |  |
|  | Avg. ± Overall SD | 5440 ± 790 |  |  |  |
| Replicate Experiment | No (Speed) Vortexing | 16800 ± 3100 | NA | 0.36 | 0.78 |
|  |  | 10200 ± 1700 |  |  |  |
|  |  | 15700 ± 3500 |  |  |  |
|  | Avg. ± Overall SD | 14200 ± 4000 |  |  |  |
|  | Half Speed Vortexing | 12400 ± 500 | 0.36 | NA | 0.61 |
|  |  | 16100 ± 1600 |  |  |  |
|  |  | 18800 ± 800 |  |  |  |
|  | Avg. ± Overall SD | 15800 ± 3000 |  |  |  |
|  | Top Speed Vortexing | 16200 ± 1400 | 0.78 | 0.61 | NA |
|  |  | 13600 ± 2400 |  |  |  |
|  |  | 14200 ± 2200 |  |  |  |
|  | Avg. ± Overall SD | 14700 ± 2200 |  |  |  |

a The average (Avg.) and standard deviation (SD) are based on triplicate dilution plating for each triplicate sample for each vortexing condition.  Total CFU are determined from a total sample volume of 25.1 mL. The Overall SD is calculated for the Avg. CFU per vortexing condition as described in the Materials and Methods section. Negative controls showed non-detect results (data not shown).

Table 2. Recovery of viable Y. pestis cells from SS inoculated at the 103-cell level (2100 ± 100 CFU/SS for Experiment 1, 1780 ± 160 CFU/SS for Experiment 2, and 1350 ± 80 CFU/SS for Experiment 3) after no-hold time and 48-hour hold time at 4°C.

| Hold Time | Sample Replicate | Total CFUs Recovereda |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Expt. 1 | Expt. 2 | Expt. 3 |  |
| 0-Hour Hold | 1 | 1390 ± 150 | 1130 ± 200 | 1110 ± 370 |  |
|  | 2 | 2000 ± 70 | 1220 ± 110 | 1500 ± 300 |  |
|  | 3 | 1600 ± 130 | 910 ± 230 | 950 ± 150 |  |
|  | 4 | - | - | 970 ± 140 |  |
|  | 5 | - | - | 1540 ± 320 |  |
|  | Avg. ± SD | 1660 ± 290 (80% ± 14%) | 1090 ± 210 (61% ± 12%) | 1210 ± 350 (90% ± 26%) |  |
| 48-Hour Hold | 1 | 780 ± 70 | 1720 ± 200 | 990 ± 210 |  |
|  | 2 | 910 ± 40 | 1330 ± 80 | 830 ± 270 |  |
|  | 3 | 580 ± 50 | 1240 ± 150 | 1060 ± 80 |  |
|  | 4 | - | - | 780 ± 230 |  |
|  | 5 | - | - | 820 ± 100 |  |
|  | Avg. ± SD | 760 ± 150 | 1430 ± 260 | 900 ± 260 |  |
|  |  | (37% ± 7%) | (80% ± 15%) | (67% ± 19%) |  |
| T-test |  | P = 0.011 | P = 0.119 | P = 0.052 |  |

a The average (Avg.) is based on triplicate samples for Experiments 1 and 2 and five replicate samples for Experiment 3.  Plate counts from replicate 100-µL aliquots were averaged and multiplied by the total volume recovered for each sample (ranging from 1.7-3.3 mL) to determine the “Total CFUs Recovered”. Each sample replicate shows the Avg. Total CFUs Recovered ± one standard deviation (SD). The Overall SD is calculated as described in the Materials and Methods section for the Avg. CFU recovered per hold-time treatment. Negative controls showed non-detect results (data not shown).

Table 3. Recovery of viable Y. pestis cells from SS inoculated at 102-cell and 103-cell levels.

| Experiment | Method | Total CFUs Recovered |  |
| --- | --- | --- | --- |
|  |  | 102-cell levela | 103-cell levelb |
| Experiment 1 | High Throughput Method | 130 | 1530 ± 80 |
|  |  | 157 | 870 ± 260 |
|  |  | 135 | 1250 ± 180 |
|  |  | 120 | 1360 ± 40 |
|  | Avg. (SD) | 136 ± 16 (40% ± 5%) | 1250 ± 290 (37% ± 9%) |
|  | Stomacher-Based Method | 137 | 1590 ± 30 |
|  |  | -- | 1580 ±110 |
|  |  | 131 | 2160 ± 20 |
|  |  | 138 | 1660 ± 440 |
|  | Avg. (SD) | 135 ± 4 (40% ± 1%) | 1780 ± 320 (52% ± 9%) |
|  |  | P = 0.99 | P = 0.046 |
| Experiment 2 | High Throughput Method | 93 | 760 ± 40 |
|  |  | 60 | 990 ± 140 |
|  |  | 67 | 940 ±180 |
|  |  | 76 | 980 ±100 |
|  | Avg. (SD) | 74 ± 14 (39% ± 7%) | 920 ±150 (48% ± 8%) |
|  | Stomacher-Based Method | 59 | 1320 ± 160 |
|  |  | 111 | 1170 ± 280 |
|  |  | 71 | 1620 ± 800 |
|  |  | 169 | 1370 ± 210 |
|  | Avg. (SD) | 103 ± 50 (54% ± 26%) | 1370 ± 260 (72% ± 10%) |
|  |  | P = 0.31 | P = 0.0057 |
| Experiment 3 | High Throughput Method | 160 | 2210 ± 160 |
|  |  | 161 | 1940 ± 260 |
|  |  | 132 | 2310 ± 230 |
|  |  | 147 | 2460 ± 230 |
|  | Avg. (SD) | 150 ± 14 (40% ± 4%) | 2230 ± 230 (60% ± 6%) |
|  | Stomacher-Based Method | 126 | 2230 ± 180 |
|  |  | 200 | 2280 ± 450 |
|  |  | 231 | 2370 ± 280 |
|  |  | 211 | 3110 ± 320 |
|  | Avg. (SD) | 192 ± 46 (52% ± 12%) | 2500 ± 460 (67% ± 12%) |
|  |  | P = 0.13 | P = 0.30 |

a For the 102-cell level, the total volume of the final concentrate was distributed across 2 to 4 plates, and the total CFU recovery was determined as the sum of the CFU for all sample plates. The average (Avg.) is based on four replicate samples for each treatment except for Expt. 1 for the “Stomacher-Based Method” which is based on triplicate samples since one sample was lost during processing.  Each experiment shows the average total CFUs recovered (Avg.) ± one standard deviation (SD) for each method. The percent recovery is relative to the CFU of the inoculum (corrected for dilution).  For the 102-cell levels, the inocula were: 340 ± 30 CFU/SS for Experiment 1; 190 ± 20 CFU/SS for Experiment 2; and 370 ± 10 CFU/SS for Experiment 3.  One negative control plate (out of 6) in the first experiment had a single colony.  All other negative controls showed non-detect results (data not shown).

b For the 103-cell level, plate counts from triplicate 100-µL aliquots were averaged and multiplied by the total volume recovered for each sample (ranging from 2.5-3.4 mL for the HTM and 2.9-6.4 mL for the stomacher method) to determine the “Total CFUs Recovered”, except for Expt. 3 for the “Stomacher-Based Method” which used four replicate plates (with 100 mL per plate) to calculate the average and standard deviation. The “SD” is the “Overall SD” described in the Materials and Methods section. The percent recovery is relative to the CFU of the inoculum (corrected for dilution). For the  103-cell level, the inocula were: 3410 ± 320 CFU/SS for Experiment 1; 1900 ± 150 CFU/SS for Experiment 2; and 3710 ± 120 CFU/SS for Experiment 3. Negative controls showed non-detect results (data not shown).

Table 4. RV-PCR analysis of two Y. pestis cell levels (220 ± 20 CFU and 19 ± 6 CFU) from the first replicate experiment of the HT SS sample processing method.

| Inoculum Level | Treatment | Sample Replicate | Avg. (SD) CTa |  | Avg. (SD)b DCT |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T0 | T24 |  |
| 220 ± 20 CFU | With ATD | 1 | 41.1 (0.7) | 27.9 (0.1) | 13.2 (0.7) |
|  |  | 2 | 39.4 (1.1) | 28.9 (0.7) | 10.5 (1.3) |
|  |  | 3 | 38.2 (0.1) | 28.3 (0.7) | 9.9 (0.7) |
|  |  | 4 | 39.6 (2.3) | 26.6 (0.1) | 13.0 (2.3) |
|  |  | Avg. (Overall SD) | 39.6 (1.6) | 27.9 (1.0) | 11.7 (1.9) |
|  | Without ATD | 1 | 41.3 (0.4) | 28.6 (0.4) | 12.7 (0.6) |
|  |  | 2 | 41.7 (0.7) | 27.4 (0.3) | 14.3 (0.8) |
|  |  | 3 | 40.7 (1.7) | 25.4 (0.6) | 15.3 (1.8) |
|  |  | 4 | 37.2 (0.6) | 26.5 (1.0) | 10.7 (1.2) |
|  |  | Avg. (Overall SD) | 40.2 (2.0) | 27.0 (1.3) | 13.2 (2.1) |
| 19 ± 6 CFU | With ATD | 1 | ND (NA) | 29.5 (0.5) | 15.5 (0.5) |
|  |  | 2 | ND (NA) | 32.4 (0.5) | 12.6 (0.5) |
|  |  | 3 | ND (NA) | 31.0 (1.6) | 14.0 (1.6) |
|  |  | 4 | ND (NA) | 29.1 (0.2) | 15.9 (0.2) |
|  |  | Avg. (Overall SD) | ND (NA) | 30.5 (1.5) | 14.5 (1.5) |
|  | Without ATD | 1 | ND (NA) | 31.9 (0.8) | 13.1 (0.8) |
|  |  | 2 | 38.7 (0.5) | 34.0 (1.4) | 4.7 (1.5) |
|  |  | 3 | ND (NA) | 31.9 (1.4) | 13.1 (1.4) |
|  |  | 4 | ND (NA) | 30.4 (0.3) | 14.6 (0.3) |
|  |  | Avg. (Overall SD) | 43.4 (2.9) | 32.0 (1.6) | 11.4 (4.2) |

a “Avg.” indicates average and “SD” indicates standard deviation of triplicate PCR replicates for each replicate sample. “ND” indicates not detected and “NA” indicates not applicable.  To calculate DCT, samples with ND were set to “45” which was the number of PCR cycles used; the standard deviation was set to “0” for these samples to calculate the “Overall SD”.  Negative controls from duplicates with and without ATD addition showed non-detect results (data not shown).

b Standard deviation for Avg. DCT calculated as the indeterminate error for the difference Avg. CT (T0) – Avg. CT (T24), that is the square root (SQRT) of the sum of each standard deviation squared, i.e., SQRT[(SD T0)2 + (SD T24)2].

Table 5. RV-PCR analysis of two Y. pestis cell levels (235 ± 22 and 22 ± 7 cells) from the second replicate experiment of the HT SS sample processing method.

| Inoculum Level | Treatment | Sample Replicate | Avg. (SD) CTa |  | Avg. (SD)b DCT |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T0 | T24 |  |
| 235 ± 22 CFU | With ATD | 1 | 40.1 (1.1) | 24.5 (0.2) | 15.6 (1.1) |
|  |  | 2 | 38.3 (0.8) | 25.0 (0.1) | 13.3 (0.8) |
|  |  | 3 | ND (NA) | 28.5 (0.2) | 16.5 (0.2) |
|  |  | 4 | ND (NA) | 24.8 (0.3) | 20.2 (0.3) |
|  |  | Avg. (Overall SD) | 42.1 (3.2) | 25.7 (1.7) | 16.4 (2.7) |
|  | Without ATD | 1 | 41.4 (0.6) | 26.0 (0.2) | 15.4 (0.6) |
|  |  | 2 | ND (NA) | 26.4 (0.3) | 18.6 (0.3) |
|  |  | 3 | 39.3 (0.1) | 25.2 (0.2) | 14.1 (0.2) |
|  |  | 4 | 40.4 (0.5) | 26.1 (0.2) | 14.2 (0.5) |
|  |  | Avg. (Overall SD) | 41.5 (2.3) | 25.9 (0.5) | 15.6 (1.9) |
| 22 ± 7 CFU | With ATD | 1 | ND (NA) | 29.4 (0.1) | 15.6 (0.1) |
|  |  | 2 | ND (NA) | 30.3 (0.1) | 14.7 (0.1) |
|  |  | 3 | 40.1 (1.1) | 33.0 (0.03) | 7.1 (1.1) |
|  |  | 4 | 38.3 (0.9) | 31.5 (0.1) | 6.8 (0.9) |
|  |  | Avg. (Overall SD) | 42.1 (3.2) | 31.1 (1.5) | 11.0 (4.3) |
|  | Without ATD | 1 | ND (NA) | 32.1 (0.1) | 12.9 (0.1) |
|  |  | 2 | ND (NA) | 34.1 (0.04) | 10.9 (0.04) |
|  |  | 3 | 40.5 (0.9) | 34.0 (0.2) | 6.5 (0.9) |
|  |  | 4 | ND (NA) | 30.1 (0.2) | 14.9 (0.2) |
|  |  | Avg. (Overall SD) | 43.9 (2.1) | 32.6 (1.7) | 11.3 (3.3) |

a “Avg.” indicates average and “SD” indicates standard deviation of triplicate PCR replicates for each replicate sample. “ND” indicates not detected and “NA” indicates not applicable.  To calculate DCT, samples with “ND” were set to “45” which was the number of PCR cycles used; the standard deviation was set to “0” for these samples to calculate the “Overall SD”. Negative controls from duplicates with and without ATD addition showed non-detect results (data not shown).

b Standard deviation for Avg. DCT calculated as the indeterminate error for the difference Avg. CT (T0) – Avg. CT (T24), that is the square root (SQRT) of the sum of the individual standard deviations squared, i.e., SQRT[(SD T0)2 + (SD T24)2].

Table 6. Recovery of viable F. tularensis cells after vortexing at different speeds.

| Experiment | Condition | Total CFUs Recovered Avg. ± SDa | Pairwise T-test P-values Compared to: |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | No Speed | Half Speed | Top Speed |
| Initial Experiment | No (Speed) Vortexing | 14200 ± 1600 | NA | 0.21 | 0.49 |
|  |  | 16100 ±1200 |  |  |  |
|  |  | 15800 ±1500 |  |  |  |
|  | Avg. ± Overall SD | 15400 ± 1200 |  |  |  |
|  | Half Speed Vortexing | 17100 ± 2300 | 0.21 | NA | 0.29 |
|  |  | 15700 ± 600 |  |  |  |
|  |  | 16400 ± 500 |  |  |  |
|  | Avg. ± Overall SD | 16400 ± 1400 |  |  |  |
|  | Top Speed Vortexing | 15500 ± 1900 | 0.49 | 0.29 | NA |
|  |  | 16300 ± 2200 |  |  |  |
|  |  | 15700 ± 200 |  |  |  |
|  | Avg. ± Overall SD | 15800 ± 1500 |  |  |  |
| Replicate Experiment | No (Speed) Vortexing | 11800 ± 320 | NA | 0.91 | 0.70 |
|  |  | 12400 ± 1200 |  |  |  |
|  |  | 11000 ± 1200 |  |  |  |
|  | Avg. ± Overall SD | 11700 ± 1100 |  |  |  |
|  | Half Speed Vortexing | 11600 ± 1700 | 0.91 | NA | 0.95 |
|  |  | 13500 ± 1000 |  |  |  |
|  |  | 10400 ± 800 |  |  |  |
|  | Avg. ± Overall SD | 11800 ± 1800 |  |  |  |
|  | Top Speed Vortexing | 11700 ± 600 | 0.70 | 0.95 | NA |
|  |  | 12000 ± 1800 |  |  |  |
|  |  | 11900 ± 1200 |  |  |  |
|  | Avg. ± Overall SD | 11900 ± 1150 |  |  |  |

a The average (Avg.) and standard deviation (SD) are based on triplicate dilution plating for each triplicate sample for each vortexing condition.  Total CFU are determined from a total sample volume of 10 mL. The Overall SD is calculated for the Avg. CFU per vortexing condition as described in the Materials and Methods section. Negative controls showed non-detect results (data not shown).

Table 7. Recovery of viable F. tularensis cells from SS inoculated at 103-cell levels (2130 ± 80 CFU/SS for Experiment 1 and 1880 ± 250 CFU/SS for Experiment 2) after 0-hour and 48-hour hold times at 4 °C.

| Hold Time | Total CFUs Recovereda |  |
| --- | --- | --- |
|  | Expt. 1 | Expt. 2 |
| 0-Hour Hold | 1800 ± 220 | 1210 ± 50 |
|  | 1710 ± 350 | 1410 ± 320 |
|  | 1780 ±120 | 1200 ± 130 |
| Avg. ± SD | 1760 ± 220 (83% ± 10%) | 1270 ± 200 (67% ± 11%) |
| 48-Hour Hold | 300 ± 90 | 310 ± 100 |
|  | 1200 ± 40 | 160 ± 0 |
|  | 510 ± 130 | 20 ± 20 |
| Avg. ± SD | 670 ± 420 | 160 ± 140 |
|  | (32% ± 20%) | (9% ± 7%) |
| T-test | P = 0.016 | P = 0.0005 |

a The average (Avg.) is based on triplicate samples for each hold-time treatment.  Plate counts from triplicate 100-µL aliquots per sample were averaged and multiplied by the total volume recovered for each sample (ranging from 2.4-3.4 mL) to determine the “Total CFUs Recovered”. Each sample replicate shows the Avg. Total CFUs Recovered ± one standard deviation (SD). The Overall SD is calculated as described in the Materials and Methods section for the Avg. CFU Recovered per hold-time treatment. Negative controls showed non-detect results (data not shown).

Table 8. Recovery of viable F. tularensis cells after 0-hour and 48-hour hold times at 4 °C in NB or PBS.

| Hold Time | Buffera |  | Sample Replicate | Total CFUs Recovered Avg. ± SDb |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Expt. 1 |  | Expt. 2 |  |
| 0-Hour Hold | NB |  | 1 | 2160 ± 850 |  | 1890 ± 590 |  |
|  |  |  | 2 | 2220 ± 970 |  | 2120 ± 0 |  |
|  |  |  | 3 | 2260 ± 460 |  | 2190 ±310 |  |
|  | Avg. ± Overall SD |  |  | 2210 ± 690 | 2070 ± 360 |  |  |
|  | PBS |  | 1 | 2490 ± 250 |  | 2420 ± 610 |  |
|  |  |  | 2 | 1990 ± 740 |  | 2790 ± 620 |  |
|  |  |  | 3 | 3130 ± 560 |  | 2360 ± 230 |  |
|  | Avg. ± Overall SD |  |  | 2540 ± 690 | 2520 ± 490 |  |  |
| 48-Hour Hold | NB |  | 1 | 1380 ± 560 |  | 1040 ± 120 |  |
|  |  |  | 2 | 1350 ± 360 |  | 940 ± 360 |  |
|  |  |  | 3 | 1480 ± 660 |  | 1520 ± 270 |  |
|  | Avg. ± Overall SD |  |  | 1400 ± 470 | 1170 ± 360 |  |  |
|  | PBS |  | 1 | 2050 ± 410 |  | 2360 ± 1210 |  |
|  |  |  | 2 | 2730 ± 180 |  | 2360 ± 330 |  |
|  |  |  | 3 | 1750 ± 410 |  | 2290 ± 60 |  |
|  | Avg. ± Overall SD |  |  | 2180 ± 530 | 2340 ± 630 |  |  |
| T-test P-values (0- vs. 48-Hours) |  |  | NB | < 0.0001 |  | 0.011 |  |
|  |  |  | PBS | 0.46 |  | 0.24 |  |

a NB = Neutralizing Buffer; PBS = Phosphate-buffered Saline.

b The average (Avg.) is based on triplicate samples for each buffer and hold-time.  Plate counts from 100 µL aliquots of a total sample volume of 10 mL .Each replicate sample shows the Avg. Total CFU ± one standard deviation (SD). The Overall SD is calculated for the Avg. CFU per hold-time condition as described in the Materials and Methods section. Inoculum levels were 2080 ± 250 CFU and 2270 ± 290 CFU for the first and second experiments, respectively. Negative controls showed non-detect results (data not shown).