Data/Metadata

Manuscript: “Evaluation of a High-Throughput Method for Processing Sponge-Stick Samples to Detect Viable, Non-Spore-Forming Biothreat Agents”

HS.402.1.1.22

PI: Sanjiv Shah, ORD-CESER-HSMMD-DCB

Glossary/Dictionary for Terms, Variables, and Acronyms Used to Present the DATA/Metadata

Yersinia pestis: The bacterium that causes plague infection.

Francisella tularensis: The bacterium that causes tularemia infection.

Spores: Spores are hardy dormant forms of bacteria that can survive harsh environmental conditions.

Non-Spore Forming: The bacteria that do not form spores and remain as vegetative cells.

Sponge-stick: A sampling device/tool to collect samples from non-porous surfaces.

SS: Sponge-stick

Aliquot: A small volume of liquid withdrawn/taken out from the total volume for analysis.

mL: Milliliter.

µL: Microliter.

CFU: Colony Forming Unit.

ATD: Arizona Test Dust.

MicroFunnel Filter: MicroFunnel Filter is a plastic assembly for membrane filter-based filtration of bacterial suspension. The membrane filter is then transferred onto a growth medium plate and incubated at an optimum growth temperature. After incubation, the colonies formed of bacterial (here, Yersinia pestis and Francisella tularensis) growth are counted to determine the number of bacteria present in a sample.

RV: Rapid Viability.

T0: Time Zero: Taking out an aliquot of culture suspension for DNA extraction at Zero hour incubation time for bacterial (Yersinia pestis) growth – no incubation, i.e., immediately after mixing the sample in a growth medium and shaking but before incubating at an optimum growth temperature.

T24: Time 24 hours: Taking out an aliquot of culture suspension for DNA extraction after 24 hours of incubation time for bacterial (Yersinia pestis) growth at an optimum growth temperature.

Polymerase: This catalyst (in this work, DNA polymerase enzyme) synthesizes DNA and amplifies (makes copies of) the DNA.

PCR:  Polymerase Chain Reaction. This is an assay used to detect presence of deoxyribo nucleic acid (DNA) of any biological organism or entity, including disease causing microorganisms/germs (including bioterrorism agents), from a sample. DNA polymerase generates copies of the original DNA. In this PCR assay, the DNA polymerase enzyme uses nucleotides (building blocks of DNA), and forward and reverse primers (short nucleotide sequence pieces within the same part of the gene that is amplified). The primers prime PCR to amplify the original DNA and continue a chain of 45 DNA amplification (PCR) cycles to make millions of copies of this small gene part. A fluorescently labeled probe (a short nucleotide sequence piece of the same gene part) is also included in the assay. The probe binds in between the forward and the reverse primers on the gene part DNA to be amplified. As the part of the target gene (DNA) undergoes amplification, this probe gets degraded/chewed up and emits fluorescence as a result of enzyme action, and it is measured by a specially designed instrument. When the fluorescence amount crosses a set threshold value at a particular PCR cycle number (Cycle Threshold, CT), it indicates the amplification of the specific target gene part, which, in turn, indicates presence of the specific biological organism or entity in a sample. In this work, it indicates the presence of Yersinia pestis cells in a sample.

CT(T0): PCR cycle threshold value for the T0 aliquot PCR analysis.

CT(T24): PCR cycle threshold value for the T24 aliquot PCR analysis.

∆CT: A change (∆) in the PCR cycle threshold (CT) number between two conditions. In the RV-PCR method, PCR assay is performed before and after incubation for bacterial growth, and the CT values are measured. The CT value of the PCR assay performed after incubation/growth (Time 24 hours, T24) is deducted from the CT value of the assay performed before incubation/growth (Time Zero, T0). This difference in the CT values is called ∆CT. For an assay positive result, the ∆CT value (number) is set to indicate a certain increase in the amount of the nucleic acid (DNA) and therefore, it has to be equal to or greater than (≥) the set value. Accordingly, the ∆CT value indicates presence of viable/live biological organism or entity in a sample. In this work, it indicates the presence of viable/live Yersinia pestis cells in a sample.

Avg: Average.

SD: Standard Deviation.

ND: Not Detected.

Avg (SD): Average (Standard Deviation) calculated from triplicate aliquots from each sample timepoint (e.g., T0, T24) DNA extract.

YC2 PCR Assay:  This assay targets and amplifies a specific segment of nucleic acid on the chromosome of Yersinia pestis.

----------------------------------------------------------------------------------------------------------------------

Data/Metadata

Tables

Table 1. Recovery of viable Y. pestis cells after vortexing at different speeds.

| Experiment | Condition | Total CFUs Recovered Avg. ± SDa | Pairwise T-test P-values Compared to: |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | No Speed | Half Speed | Top Speed |
| Initial Experiment | No (Speed) Vortexing | 5610 ± 1510 | NA | 0.58 | 0.92 |
|  |  | 5770 ± 440 |  |  |  |
|  |  | 5100 ± 2190 |  |  |  |
|  | Avg. ± Overall SD | 5500 ± 1380 |  |  |  |
|  | Half Speed Vortexing | 7530 ± 1400 | 0.58 | NA | 0.76 |
|  |  | 3350 ± 950 |  |  |  |
|  |  | 4180 ± 150 |  |  |  |
|  | Avg. ± Overall SD | 5020 ± 2100 |  |  |  |
|  | Top Speed Vortexing | 5190 ± 810 | 0.92 | 0.76 | NA |
|  |  | 5690 ± 730 |  |  |  |
|  |  | 5440 ± 1050 |  |  |  |
|  | Avg. ± Overall SD | 5440 ± 790 |  |  |  |
| Replicate Experiment | No (Speed) Vortexing | 16800 ± 3100 | NA | 0.36 | 0.78 |
|  |  | 10200 ± 1700 |  |  |  |
|  |  | 15700 ± 3500 |  |  |  |
|  | Avg. ± Overall SD | 14200 ± 4000 |  |  |  |
|  | Half Speed Vortexing | 12400 ± 500 | 0.36 | NA | 0.61 |
|  |  | 16100 ± 1600 |  |  |  |
|  |  | 18800 ± 800 |  |  |  |
|  | Avg. ± Overall SD | 15800 ± 3000 |  |  |  |
|  | Top Speed Vortexing | 16200 ± 1400 | 0.78 | 0.61 | NA |
|  |  | 13600 ± 2400 |  |  |  |
|  |  | 14200 ± 2200 |  |  |  |
|  | Avg. ± Overall SD | 14700 ± 2200 |  |  |  |

a The average (Avg.) and standard deviation (SD) are based on triplicate dilution plating for each triplicate sample for each vortexing condition.  Total CFU are determined from a total sample volume of 25.1 mL. The Overall SD is calculated for the Avg. CFU per vortexing condition as described in the Materials and Methods section. Negative controls showed non-detect results (data not shown).

Table 2. Recovery of viable Y. pestis cells from SS inoculated at the 103-cell level (2100 ± 100 CFU/SS for Experiment 1, 1780 ± 160 CFU/SS for Experiment 2, and 1350 ± 80 CFU/SS for Experiment 3) after no-hold time and 48-hour hold time at 4°C.

| Hold Time | Sample Replicate | Total CFUs Recovereda |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Expt. 1 | Expt. 2 | Expt. 3 |  |
| 0-Hour Hold | 1 | 1390 ± 150 | 1130 ± 200 | 1110 ± 370 |  |
|  | 2 | 2000 ± 70 | 1220 ± 110 | 1500 ± 300 |  |
|  | 3 | 1600 ± 130 | 910 ± 230 | 950 ± 150 |  |
|  | 4 | - | - | 970 ± 140 |  |
|  | 5 | - | - | 1540 ± 320 |  |
|  | Avg. ± SD | 1660 ± 290 (80% ± 14%) | 1090 ± 210 (61% ± 12%) | 1210 ± 350 (90% ± 26%) |  |
| 48-Hour Hold | 1 | 780 ± 70 | 1720 ± 200 | 990 ± 210 |  |
|  | 2 | 910 ± 40 | 1330 ± 80 | 830 ± 270 |  |
|  | 3 | 580 ± 50 | 1240 ± 150 | 1060 ± 80 |  |
|  | 4 | - | - | 780 ± 230 |  |
|  | 5 | - | - | 820 ± 100 |  |
|  | Avg. ± SD | 760 ± 150 | 1430 ± 260 | 900 ± 260 |  |
|  |  | (37% ± 7%) | (80% ± 15%) | (67% ± 19%) |  |
| T-test |  | P = 0.011 | P = 0.119 | P = 0.052 |  |

a The average (Avg.) is based on triplicate samples for Experiments 1 and 2 and five replicate samples for Experiment 3.  Plate counts from replicate 100-µL aliquots were averaged and multiplied by the total volume recovered for each sample (ranging from 1.7-3.3 mL) to determine the “Total CFUs Recovered”. Each sample replicate shows the Avg. Total CFUs Recovered ± one standard deviation (SD). The Overall SD is calculated as described in the Materials and Methods section for the Avg. CFU recovered per hold-time treatment. Negative controls showed non-detect results (data not shown).

Table 3. Recovery of viable Y. pestis cells from SS inoculated at 102-cell and 103-cell levels.

| Experiment | Method | Total CFUs Recovered |  |
| --- | --- | --- | --- |
|  |  | 102-cell levela | 103-cell levelb |
| Experiment 1 | High Throughput Method | 130 | 1530 ± 80 |
|  |  | 157 | 870 ± 260 |
|  |  | 135 | 1250 ± 180 |
|  |  | 120 | 1360 ± 40 |
|  | Avg. (SD) | 136 ± 16 (40% ± 5%) | 1250 ± 290 (37% ± 9%) |
|  | Stomacher-Based Method | 137 | 1590 ± 30 |
|  |  | -- | 1580 ±110 |
|  |  | 131 | 2160 ± 20 |
|  |  | 138 | 1660 ± 440 |
|  | Avg. (SD) | 135 ± 4 (40% ± 1%) | 1780 ± 320 (52% ± 9%) |
|  |  | P = 0.99 | P = 0.046 |
| Experiment 2 | High Throughput Method | 93 | 760 ± 40 |
|  |  | 60 | 990 ± 140 |
|  |  | 67 | 940 ±180 |
|  |  | 76 | 980 ±100 |
|  | Avg. (SD) | 74 ± 14 (39% ± 7%) | 920 ±150 (48% ± 8%) |
|  | Stomacher-Based Method | 59 | 1320 ± 160 |
|  |  | 111 | 1170 ± 280 |
|  |  | 71 | 1620 ± 800 |
|  |  | 169 | 1370 ± 210 |
|  | Avg. (SD) | 103 ± 50 (54% ± 26%) | 1370 ± 260 (72% ± 10%) |
|  |  | P = 0.31 | P = 0.0057 |
| Experiment 3 | High Throughput Method | 160 | 2210 ± 160 |
|  |  | 161 | 1940 ± 260 |
|  |  | 132 | 2310 ± 230 |
|  |  | 147 | 2460 ± 230 |
|  | Avg. (SD) | 150 ± 14 (40% ± 4%) | 2230 ± 230 (60% ± 6%) |
|  | Stomacher-Based Method | 126 | 2230 ± 180 |
|  |  | 200 | 2280 ± 450 |
|  |  | 231 | 2370 ± 280 |
|  |  | 211 | 3110 ± 320 |
|  | Avg. (SD) | 192 ± 46 (52% ± 12%) | 2500 ± 460 (67% ± 12%) |
|  |  | P = 0.13 | P = 0.30 |

a For the 102-cell level, the total volume of the final concentrate was distributed across 2 to 4 plates, and the total CFU recovery was determined as the sum of the CFU for all sample plates. The average (Avg.) is based on four replicate samples for each treatment except for Expt. 1 for the “Stomacher-Based Method” which is based on triplicate samples since one sample was lost during processing.  Each experiment shows the average total CFUs recovered (Avg.) ± one standard deviation (SD) for each method. The percent recovery is relative to the CFU of the inoculum (corrected for dilution).  For the 102-cell levels, the inocula were: 340 ± 30 CFU/SS for Experiment 1; 190 ± 20 CFU/SS for Experiment 2; and 370 ± 10 CFU/SS for Experiment 3.  One negative control plate (out of 6) in the first experiment had a single colony.  All other negative controls showed non-detect results (data not shown).

b For the 103-cell level, plate counts from triplicate 100-µL aliquots were averaged and multiplied by the total volume recovered for each sample (ranging from 2.5-3.4 mL for the HTM and 2.9-6.4 mL for the stomacher method) to determine the “Total CFUs Recovered”, except for Expt. 3 for the “Stomacher-Based Method” which used four replicate plates (with 100 mL per plate) to calculate the average and standard deviation. The “SD” is the “Overall SD” described in the Materials and Methods section. The percent recovery is relative to the CFU of the inoculum (corrected for dilution). For the  103-cell level, the inocula were: 3410 ± 320 CFU/SS for Experiment 1; 1900 ± 150 CFU/SS for Experiment 2; and 3710 ± 120 CFU/SS for Experiment 3. Negative controls showed non-detect results (data not shown).

Table 4. RV-PCR analysis of two Y. pestis cell levels (220 ± 20 CFU and 19 ± 6 CFU) from the first replicate experiment of the HT SS sample processing method.

| Inoculum Level | Treatment | Sample Replicate | Avg. (SD) CTa |  | Avg. (SD)b DCT |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T0 | T24 |  |
| 220 ± 20 CFU | With ATD | 1 | 41.1 (0.7) | 27.9 (0.1) | 13.2 (0.7) |
|  |  | 2 | 39.4 (1.1) | 28.9 (0.7) | 10.5 (1.3) |
|  |  | 3 | 38.2 (0.1) | 28.3 (0.7) | 9.9 (0.7) |
|  |  | 4 | 39.6 (2.3) | 26.6 (0.1) | 13.0 (2.3) |
|  |  | Avg. (Overall SD) | 39.6 (1.6) | 27.9 (1.0) | 11.7 (1.9) |
|  | Without ATD | 1 | 41.3 (0.4) | 28.6 (0.4) | 12.7 (0.6) |
|  |  | 2 | 41.7 (0.7) | 27.4 (0.3) | 14.3 (0.8) |
|  |  | 3 | 40.7 (1.7) | 25.4 (0.6) | 15.3 (1.8) |
|  |  | 4 | 37.2 (0.6) | 26.5 (1.0) | 10.7 (1.2) |
|  |  | Avg. (Overall SD) | 40.2 (2.0) | 27.0 (1.3) | 13.2 (2.1) |
| 19 ± 6 CFU | With ATD | 1 | ND (NA) | 29.5 (0.5) | 15.5 (0.5) |
|  |  | 2 | ND (NA) | 32.4 (0.5) | 12.6 (0.5) |
|  |  | 3 | ND (NA) | 31.0 (1.6) | 14.0 (1.6) |
|  |  | 4 | ND (NA) | 29.1 (0.2) | 15.9 (0.2) |
|  |  | Avg. (Overall SD) | ND (NA) | 30.5 (1.5) | 14.5 (1.5) |
|  | Without ATD | 1 | ND (NA) | 31.9 (0.8) | 13.1 (0.8) |
|  |  | 2 | 38.7 (0.5) | 34.0 (1.4) | 4.7 (1.5) |
|  |  | 3 | ND (NA) | 31.9 (1.4) | 13.1 (1.4) |
|  |  | 4 | ND (NA) | 30.4 (0.3) | 14.6 (0.3) |
|  |  | Avg. (Overall SD) | 43.4 (2.9) | 32.0 (1.6) | 11.4 (4.2) |

a “Avg.” indicates average and “SD” indicates standard deviation of triplicate PCR replicates for each replicate sample. “ND” indicates not detected and “NA” indicates not applicable.  To calculate DCT, samples with ND were set to “45” which was the number of PCR cycles used; the standard deviation was set to “0” for these samples to calculate the “Overall SD”.  Negative controls from duplicates with and without ATD addition showed non-detect results (data not shown).

b Standard deviation for Avg. DCT calculated as the indeterminate error for the difference Avg. CT (T0) – Avg. CT (T24), that is the square root (SQRT) of the sum of each standard deviation squared, i.e., SQRT[(SD T0)2 + (SD T24)2].

Table 5. RV-PCR analysis of two Y. pestis cell levels (235 ± 22 and 22 ± 7 cells) from the second replicate experiment of the HT SS sample processing method.

| Inoculum Level | Treatment | Sample Replicate | Avg. (SD) CTa |  | Avg. (SD)b DCT |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T0 | T24 |  |
| 235 ± 22 CFU | With ATD | 1 | 40.1 (1.1) | 24.5 (0.2) | 15.6 (1.1) |
|  |  | 2 | 38.3 (0.8) | 25.0 (0.1) | 13.3 (0.8) |
|  |  | 3 | ND (NA) | 28.5 (0.2) | 16.5 (0.2) |
|  |  | 4 | ND (NA) | 24.8 (0.3) | 20.2 (0.3) |
|  |  | Avg. (Overall SD) | 42.1 (3.2) | 25.7 (1.7) | 16.4 (2.7) |
|  | Without ATD | 1 | 41.4 (0.6) | 26.0 (0.2) | 15.4 (0.6) |
|  |  | 2 | ND (NA) | 26.4 (0.3) | 18.6 (0.3) |
|  |  | 3 | 39.3 (0.1) | 25.2 (0.2) | 14.1 (0.2) |
|  |  | 4 | 40.4 (0.5) | 26.1 (0.2) | 14.2 (0.5) |
|  |  | Avg. (Overall SD) | 41.5 (2.3) | 25.9 (0.5) | 15.6 (1.9) |
| 22 ± 7 CFU | With ATD | 1 | ND (NA) | 29.4 (0.1) | 15.6 (0.1) |
|  |  | 2 | ND (NA) | 30.3 (0.1) | 14.7 (0.1) |
|  |  | 3 | 40.1 (1.1) | 33.0 (0.03) | 7.1 (1.1) |
|  |  | 4 | 38.3 (0.9) | 31.5 (0.1) | 6.8 (0.9) |
|  |  | Avg. (Overall SD) | 42.1 (3.2) | 31.1 (1.5) | 11.0 (4.3) |
|  | Without ATD | 1 | ND (NA) | 32.1 (0.1) | 12.9 (0.1) |
|  |  | 2 | ND (NA) | 34.1 (0.04) | 10.9 (0.04) |
|  |  | 3 | 40.5 (0.9) | 34.0 (0.2) | 6.5 (0.9) |
|  |  | 4 | ND (NA) | 30.1 (0.2) | 14.9 (0.2) |
|  |  | Avg. (Overall SD) | 43.9 (2.1) | 32.6 (1.7) | 11.3 (3.3) |

a “Avg.” indicates average and “SD” indicates standard deviation of triplicate PCR replicates for each replicate sample. “ND” indicates not detected and “NA” indicates not applicable.  To calculate DCT, samples with “ND” were set to “45” which was the number of PCR cycles used; the standard deviation was set to “0” for these samples to calculate the “Overall SD”. Negative controls from duplicates with and without ATD addition showed non-detect results (data not shown).

b Standard deviation for Avg. DCT calculated as the indeterminate error for the difference Avg. CT (T0) – Avg. CT (T24), that is the square root (SQRT) of the sum of the individual standard deviations squared, i.e., SQRT[(SD T0)2 + (SD T24)2].

Table 6. Recovery of viable F. tularensis cells after vortexing at different speeds.

| Experiment | Condition | Total CFUs Recovered Avg. ± SDa | Pairwise T-test P-values Compared to: |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | No Speed | Half Speed | Top Speed |
| Initial Experiment | No (Speed) Vortexing | 14200 ± 1600 | NA | 0.21 | 0.49 |
|  |  | 16100 ±1200 |  |  |  |
|  |  | 15800 ±1500 |  |  |  |
|  | Avg. ± Overall SD | 15400 ± 1200 |  |  |  |
|  | Half Speed Vortexing | 17100 ± 2300 | 0.21 | NA | 0.29 |
|  |  | 15700 ± 600 |  |  |  |
|  |  | 16400 ± 500 |  |  |  |
|  | Avg. ± Overall SD | 16400 ± 1400 |  |  |  |
|  | Top Speed Vortexing | 15500 ± 1900 | 0.49 | 0.29 | NA |
|  |  | 16300 ± 2200 |  |  |  |
|  |  | 15700 ± 200 |  |  |  |
|  | Avg. ± Overall SD | 15800 ± 1500 |  |  |  |
| Replicate Experiment | No (Speed) Vortexing | 11800 ± 320 | NA | 0.91 | 0.70 |
|  |  | 12400 ± 1200 |  |  |  |
|  |  | 11000 ± 1200 |  |  |  |
|  | Avg. ± Overall SD | 11700 ± 1100 |  |  |  |
|  | Half Speed Vortexing | 11600 ± 1700 | 0.91 | NA | 0.95 |
|  |  | 13500 ± 1000 |  |  |  |
|  |  | 10400 ± 800 |  |  |  |
|  | Avg. ± Overall SD | 11800 ± 1800 |  |  |  |
|  | Top Speed Vortexing | 11700 ± 600 | 0.70 | 0.95 | NA |
|  |  | 12000 ± 1800 |  |  |  |
|  |  | 11900 ± 1200 |  |  |  |
|  | Avg. ± Overall SD | 11900 ± 1150 |  |  |  |

a The average (Avg.) and standard deviation (SD) are based on triplicate dilution plating for each triplicate sample for each vortexing condition.  Total CFU are determined from a total sample volume of 10 mL. The Overall SD is calculated for the Avg. CFU per vortexing condition as described in the Materials and Methods section. Negative controls showed non-detect results (data not shown).

Table 7. Recovery of viable F. tularensis cells from SS inoculated at 103-cell levels (2130 ± 80 CFU/SS for Experiment 1 and 1880 ± 250 CFU/SS for Experiment 2) after 0-hour and 48-hour hold times at 4 °C.

| Hold Time | Total CFUs Recovereda |  |
| --- | --- | --- |
|  | Expt. 1 | Expt. 2 |
| 0-Hour Hold | 1800 ± 220 | 1210 ± 50 |
|  | 1710 ± 350 | 1410 ± 320 |
|  | 1780 ±120 | 1200 ± 130 |
| Avg. ± SD | 1760 ± 220 (83% ± 10%) | 1270 ± 200 (67% ± 11%) |
| 48-Hour Hold | 300 ± 90 | 310 ± 100 |
|  | 1200 ± 40 | 160 ± 0 |
|  | 510 ± 130 | 20 ± 20 |
| Avg. ± SD | 670 ± 420 | 160 ± 140 |
|  | (32% ± 20%) | (9% ± 7%) |
| T-test | P = 0.016 | P = 0.0005 |

a The average (Avg.) is based on triplicate samples for each hold-time treatment.  Plate counts from triplicate 100-µL aliquots per sample were averaged and multiplied by the total volume recovered for each sample (ranging from 2.4-3.4 mL) to determine the “Total CFUs Recovered”. Each sample replicate shows the Avg. Total CFUs Recovered ± one standard deviation (SD). The Overall SD is calculated as described in the Materials and Methods section for the Avg. CFU Recovered per hold-time treatment. Negative controls showed non-detect results (data not shown).

Table 8. Recovery of viable F. tularensis cells after 0-hour and 48-hour hold times at 4 °C in NB or PBS.

| Hold Time | Buffera |  | Sample Replicate | Total CFUs Recovered Avg. ± SDb |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Expt. 1 |  | Expt. 2 |  |
| 0-Hour Hold | NB |  | 1 | 2160 ± 850 |  | 1890 ± 590 |  |
|  |  |  | 2 | 2220 ± 970 |  | 2120 ± 0 |  |
|  |  |  | 3 | 2260 ± 460 |  | 2190 ±310 |  |
|  | Avg. ± Overall SD |  |  | 2210 ± 690 | 2070 ± 360 |  |  |
|  | PBS |  | 1 | 2490 ± 250 |  | 2420 ± 610 |  |
|  |  |  | 2 | 1990 ± 740 |  | 2790 ± 620 |  |
|  |  |  | 3 | 3130 ± 560 |  | 2360 ± 230 |  |
|  | Avg. ± Overall SD |  |  | 2540 ± 690 | 2520 ± 490 |  |  |
| 48-Hour Hold | NB |  | 1 | 1380 ± 560 |  | 1040 ± 120 |  |
|  |  |  | 2 | 1350 ± 360 |  | 940 ± 360 |  |
|  |  |  | 3 | 1480 ± 660 |  | 1520 ± 270 |  |
|  | Avg. ± Overall SD |  |  | 1400 ± 470 | 1170 ± 360 |  |  |
|  | PBS |  | 1 | 2050 ± 410 |  | 2360 ± 1210 |  |
|  |  |  | 2 | 2730 ± 180 |  | 2360 ± 330 |  |
|  |  |  | 3 | 1750 ± 410 |  | 2290 ± 60 |  |
|  | Avg. ± Overall SD |  |  | 2180 ± 530 | 2340 ± 630 |  |  |
| T-test P-values (0- vs. 48-Hours) |  |  | NB | < 0.0001 |  | 0.011 |  |
|  |  |  | PBS | 0.46 |  | 0.24 |  |

a NB = Neutralizing Buffer; PBS = Phosphate-buffered Saline.

b The average (Avg.) is based on triplicate samples for each buffer and hold-time.  Plate counts from 100 µL aliquots of a total sample volume of 10 mL. Each replicate sample shows the Avg. Total CFU ± one standard deviation (SD). The Overall SD is calculated for the Avg. CFU per hold-time condition as described in the Materials and Methods section. Inoculum levels were 2080 ± 250 CFU and 2270 ± 290 CFU for the first and second experiments, respectively. Negative controls showed non-detect results (data not shown).