Culturing cells on microcarriers

Chemicals and Supplies

Minimum Essential Medium (MEM) with Hanks' balanced salts

0.25% Trypsin-EDTA

Penicillin-Streptomycin (PS, 10,000 U/mL)

N-2-hydroxyethylpiperazine-N-2-ethane sulfonic acid (HEPES, 1 M)

Heat inactivated fetal bovine serum

Glutamine 100X (200 mM)

Phosphate buffer saline

Ultrapure H2O (molecular biology grade)

Cell culture flasks (T225, T75, T25)

Normocin (50 mg/mL)

Pipet controller

Serological pipets (5 mL, 10 mL, 25 mL)

Centrifuge tubes (15 mL, 50 mL)

MEM (10X, with Hanks' balanced salts)

Sodium bicarbonate (cell culture grade)

Glucose (cell culture grade)

Microcarriers (Corning 3781 and 3779)

99.8% D2O

Slow-speed magnetic stirrer

Stirrer controller

Bioreactor (125 mL Reusable Glass Spinner)

Cell counter

Cell culture incubator

Biosafety Cabinet

Microscope with a 20X objective

Cell Culturing.

To culture HepG2 cells (ATCC, Manassas, VA) without CO2, we used Minimum Essential Medium (MEM) with Hanks' balanced salts (Fisher Scientific), supplemented with 10% (v/v) heat inactivated fetal bovine serum (HI FBS, Atlanta Biologicals), 100 units/mL penicillin, 100 μg/mL streptomycin (PS, Fisher Scientific), 100 μg/mL normocin (InvivoGen), 4 g/L glucose (Sigma) and 20 mM N-2-hydroxyethylpiperazine-N-2-ethane sulfonic acid (HEPES, Fisher Scientific)18. Specifically, HepG2 cells were cultivated at 37 °C in a humidified atmosphere (without CO2) as monolayer cultures in a T225 flask (Fisher Scientific) with 40 mL MEM growth medium (Table 1). The medium was refreshed twice weekly (washed with 15 mL phosphate buffered saline (PBS, Fisher Scientific)). After reaching 90% confluence, the cells were harvested using 0.25% trypsin-EDTA (1X) (Fisher Scientific) and counted using a cell counter (Countess 3, Invitrogen).

Culturing HepG2 cells on microcarriers.

- In a biosafety cabinet (BSC), add polystyrene microcarriers into a 15 mL centrifuge tube to 1.8 mL (1.2g. 1.5 mL of dry microcarrier weighs 1g).

- Add 5 mL PBS into the 15 mL tube and shake the tube.

- Hydrate microcarriers for at least 20 min in a BSC or overnight in a fridge.

- Remove the PBS from the 15 mL tube.

- Add 5 mL no-HEPES MEM growth medium (Table 2) into the 15 mL tube containing microcarriers.

- Transfer microcarriers/growth medium mixture into a T75 flask.

- Add 40 mL no-HEPES MEM growth medium into the T75 flask.

- Harvest HepG2 cells using 0.25% Trypsin-EDTA.

- Transfer 1x107 cells into the T75 flask containing microcarriers.

- Incubate the T75 flask containing HepG2 cells and microcarriers at 37 °C in a humidified atmosphere (without CO2) overnight. Note: Do not touch the T75 flask during this incubation period.

- After 20 hours, in a BSC, remove 0.1 mL of the cells/microcarriers mixture from bottom of the flask and combined in a 6 cm dish with 1 mL medium taken from the top portion of the medium in the flask.

- Determined the percent confluence of the cells on microcarriers using an inverted microscope (Olympus) equipped with a 20X objective.

- Record images of microcarrier-cultured cells using cellSens software (Olympus).

  - Setup the microscope normally and focus on the section you want to photograph.

  - Double click on the program “cellSens Standard”

  - Click on the “Live” button in the upper right-hand corner.

  - Change the microscope shutter switch so that it is on camera.

  - Click on the “Snap” button to capture cell image.

  - Save the image.

- Place the T75 flask of cell-adhered microcarriers upright for 3 minutes to allow the microcarriers to settle.

- Aspirate the medium out of T75 flash using a 10 mL serological pipet.

- Add 25 mL of 16% D2O cell growth medium (Table 4) to the T75 flask.

- Repeat the above steps once (15 and 16).

- Transfer cell-adhered microcarriers and medium into a bioreactor (Corning).

- Place the bioreactor on the slow-speed magnetic stirrer inside a cell culture incubator and incubate the cells at 37 °C in a humidified atmosphere with stir rate of 30 RPM.

- MCF7, LN229 and DU145 cells were cultured in the same way.

Table 1. MEM growth medium (4 g/L glucose)

| Component | Volume (mL) |
| --- | --- |
| MEM with Hanks’ balanced salts | 500 |
| HI FBS | 58 |
| Normocin (50 mg/mL) | 1.2 |
| PS | 5.8 |
| Glucose (0.5 g/mL) | 3.6 |
| HEPES (1 M) | 11.6 |

Table 2. No-HEPES MEM growth medium (4 g/L glucose)

| Component | Volume (mL) |
| --- | --- |
| MEM with Hanks’ balanced salts | 500 |
| HI FBS | 57 |
| Normocin (50 mg/mL) | 1.2 |
| PS | 5.7 |
| Glucose (0.5 g/mL) | 3.5 |

Table 3. 75% D2O no-HEPES MEM growth medium (4 g/L glucose)

| Component | Volume (mL) |
| --- | --- |
| MEM 10X | 10 |
| 99.8% D2O | 75.15 |
| Glutamine 100X (200 mM) | 1 |
| Glucose (0.5 g/mL) | 0.6 |
| HI FBS | 10 |
| NaHCO3 (1 M) | 0.42 |
| PS | 1 |
| Normocin (50 mg/mL) | 0.2 |
| Ultrapure H2O (molecular biology grade) | 1.63 |
| Total volume | 100 |

Table 4. 16% D2O no-HEPES MEM growth medium (4 g/L glucose)

| Component | Volume (mL) |
| --- | --- |
| 75% D2O no-HEPES MEM growth medium (Table 3) | 10 |
| no-HEPES MEM growth medium (Table 2) | 36.875 |

Table 5. 15% D2O no-HEPES MEM growth medium (4 g/L glucose)

| Component | Volume (mL) |
| --- | --- |
| 75% D2O no-HEPES MEM growth medium (Table 3) | 10 |
| no-HEPES MEM growth medium (Table 2) | 40 |