Sample preparation for NMR spectroscopy

Chemicals and Supplies

Minimum Essential Medium (MEM) with Hanks' balanced salts

Penicillin-Streptomycin (PS, 10,000 U/mL)

Heat inactivated fetal bovine serum

Glutamine 100X (200 mM)

Phosphate buffer saline

Ultrapure H2O (molecular biology grade)

Cell culture flasks (T225, T75 and T25)

Normocin (50 mg/mL)

Pipet controller

Serological pipets (5 mL, 10 mL and 25 mL)

Centrifuge tubes (15 mL and 50 mL)

MEM (10X, with Hanks' balanced salts)

Sodium bicarbonate (cell culture grade)

Glucose (cell culture grade)

Metformin

Succinate

99.95% D2O

Microcarriers (Corning 3781 and 3779)

99.8% D2O

Sodium 2,2-dimethyl-2-silapentane-5-sulfonate-d6 (DSS)

Screwcap 5 mm NMR tubes

5 mm NMR tubes

Long-tip Pasteur pipettes

Capillary tubes (1.7 mm OD x 100 mm long)

Capillary tube caps

Extraction rods

α-Fe2O3 nanorods aqueous dispersion (20% w/w, US Research Nanomaterials)

Slow-speed magnetic stirrer

Stirrer controller

Bioreactor (125 mL Reusable Glass Spinner)

Cell counter

Cell culture incubator

Biosafety Cabinet

Microscope with a 20X objective

Sample preparation

- Move the bioreactor containing cell-adhered microcarriers from the incubator to a biosafety cabinet (BSC).

- Examine the confluence of the cells on microcarriers (See SOP Culturing cells on microcarriers, steps 11-13).

- After settling (~2 minutes), aspirate the medium (~50 mL) in the bioreactor into a 50 mL centrifuge tube.

- Transferred half of the cell-adhererd microcarriers into a 15 mL centrifuge tube.

- Add the medium in the 50 mL tube back to the bioreactor.

- Add 10 mL 16% D2O growth medium (Table 4) into the bioreactor.

- Place the bioreactor on the stirrer in the cell culture incubator and incubate the cells at 37 °C in a humidified atmosphere with stir rate of 30 RPM.

- After settling (~1 mL of cell-adhered microcarriers), remove the top portion (i.e., void of cell-adhered microcarriers) of the medium in 15 mL centrifuge tube (see step 3).

- Add 8 mL 37 °C no-HEPES/no D2O medium (Table 2) supplemented 0.01% (w/v) α-Fe2O3 nanorods (US Research Nanomaterials) to the 15 mL tube.

- After settling, remove about 6 mL of the medium.

- Transfer the cell-adhered microcarriers and medium mixture into a 5 mm screw-cap NMR tube using a 5 mL serological pipet attached to a pipette controller (Drummond Scientific), 0.5 mL each time.

- Place the NMR tube in an incubator at 37 °C for 3 minutes.

- In the BSC, aspirate the top portion of the medium from the NMR tube using a long-tip Pasteur pipet.

- Add 1 mL of 16% D2O no-HEPES medium (Table 4) supplemented with 0.01% (w/v) α-Fe2O3 nanorods into the NMR tube.

- Aspirate the medium from the bottom of the NMR tube to pull fresh medium through the cell-adhered microcarriers. Note that sufficient medium must be retained in the NMR tube to ensure complete immersion of the cell-bearing microcarriers.

- Repeat the above two steps once (adding/removing 1 mL 16% D2O medium supplemented with 0.01% (w/v) α-Fe2O3 nanorods).

- Add 2 mL of the above 16% D2O medium (step 13) into the NMR tube and aspirate the excess amount of the medium from the bottom of the NMR tube until the meniscus of the medium is at the bottom neck of the air reservoir.

- Place the NMR tube in the incubator at 37 °C for 5 minutes before the sample is transported to the NMR facility. Note that the sample must be kept in upright position during sample transport.

- Samples containing different concentrations of metformin (or other chemicals) can be prepared as above but supplemented with appropriate amounts of a 1 M metformin stock solution (0.1 M) in 16% D2O.

Calculation of D2O% for produce neutral buoyancy of microcarriers (Corning)

where PD2O% is the percentage of D2O, and Dm, DD2O(MEM), DH2O(MEM), DD2O, and DH2O are the densities of the bare microcarriers, 100% D2O cell growth medium, 100% H2O cell growth medium, D2O and H2O, respectively. The difference between the densities DD2O(MEM) and DH2O(MEM) is equal to the difference in the densities DD2O (1.11 g/mL) and DH2O (1.0 g/mL). The specified lower range of Dm and the measured DH2O(MEM) are 1.022 g/mL and 1.005 g/mL, respectively. Thus, the calculated percentage of D2O required to generate neutral buoyancy for bare microcarriers is 15.5%:

Because a distribution of densities is present in a population of microcarriers, this calculated value was then fine adjusted with using 100% H2O growth medium. D2O percentage of 15±0.1% was found to be suitable to account for the lower density fraction of microcarriers. Use of 16±0.1% D2O in the cell growth medium was necessary to achieve neutral buoyancy of the cell-adhered microcarriers.

Table 1. MEM growth medium (4 g/L glucose)

| Component | Volume (mL) |
| --- | --- |
| MEM with Hanks’ balanced salts | 500 |
| HI FBS | 58 |
| Normocin (50 mg/mL) | 1.2 |
| PS | 5.8 |
| Glucose (0.5 g/mL) | 3.6 |
| HEPES (1 M) | 11.6 |

Table 2. No-HEPES MEM growth medium (4 g/L glucose)

| Component | Volume (mL) |
| --- | --- |
| MEM with Hanks’ balanced salts | 500 |
| HI FBS | 57 |
| Normocin (50 mg/mL) | 1.2 |
| PS | 5.7 |
| Glucose (0.5 g/mL) | 3.5 |

Table 3. 75% D2O no-HEPES MEM growth medium (4 g/L glucose)

| Component | Volume (mL) |
| --- | --- |
| MEM 10X | 10 |
| 99.8% D2O | 75.15 |
| Glutamine 100X (200 mM) | 1 |
| Glucose (0.5 g/mL) | 0.6 |
| HI FBS | 10 |
| NaHCO3 (1 M) | 0.42 |
| PS | 1 |
| Normocin (50 mg/mL) | 0.2 |
| Ultrapure H2O (molecular biology grade) | 1.63 |
| Total volume | 100 |

Table 4. 16% D2O no-HEPES MEM growth medium (4 g/L glucose)

| Component | Volume (mL) |
| --- | --- |
| 75% D2O no-HEPES MEM growth medium (Table 3) | 10 |
| no-HEPES MEM growth medium (Table 2) | 36.875 |

Table 5. 15% D2O no-HEPES MEM growth medium (4 g/L glucose)

| Component | Volume (mL) |
| --- | --- |
| 75% D2O no-HEPES MEM growth medium (Table 3) | 10 |
| no-HEPES MEM growth medium (Table 2) | 40 |